| Roll No. |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

PRESIDENCY UNIVERSITY

Bengaluru

End - Term Examinations – JANUARY 2025

Date: 08/01/2025 Time: 09:30 am – 12:30 pm

| School: SOE | Program: B.Tech - EEE |  |
| --- | --- | --- |
| Course Code : EEE2012 | Course Name : Electrical & Electronic measurements and Instrumentation |  |
| Semester: V | Max Marks: 100 | Weightage: 50% |

| CO - Levels | CO1 | CO2 | CO3 | CO4 |
| --- | --- | --- | --- | --- |
| Marks | 8 | 22 | 44 | 26 |

Instructions:

- Read all questions carefully and answer accordingly.

- Do not write anything on the question paper other than roll number.

Part A

| Answer ALL the Questions. Each question carries 2marks. 10Q x 2M=20M |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | Define the concept of hysteresis in meters. | 2 Marks | L1 | CO1 |
| 2 | What is the purpose of statistical evaluation in measurement data? | 2 Marks | L1 | CO1 |
| 3 | Recall the concept of True value and Measured value. | 2 Marks | L1 | CO1 |
| 4 | What is the purpose of springs in PMMC instrument? | 2 Marks | L1 | CO2 |
| 5 | What do you understand by the term Gaussian distribution or normal distribution? | 2 Marks | L1 | CO1 |
| 6 | What is a transducer? | 2 Marks | L1 | CO4 |
| 7 | Recall the concept of telemetry | 2 Marks | L1 | CO4 |
| 8 | List any 2 parts of a 3 digit DVM | 2 Marks | L1 | CO3 |
| 9 | Tell the difference between a C.T and P.T | 2 Marks | L1 | CO4 |
| 10 | Find the resolution of a 4 digit multimeter on a 1 volt range. | 2 Marks | L1 | CO3 |

Part B

| Answer the Questions Total 80 Marks. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 11. | a. | Explain the concept of 4 digit DVM in a proper sequence. | 10 Marks | L2 | CO3 |
|  | b. | Explain the concept of vertical deflection and horizontal deflection in a DSO with the help of an example. | 10 Marks | L2 | CO3 |
| or |  |  |  |  |  |
| 12. | a. | Summarize the working of a smart meter. | 10 Marks | L2 | CO3 |
|  | b. | Summarize the working of an electromechanical Energy meter. | 10 Marks | L2 | CO3 |
| 13. | a. | Explain the classification of transducers based on the nature of the input and output signals. Discuss the factors that should be considered when selecting a transducer for a specific application. | 10 Marks | L2 | CO4 |
|  | b. | Demonstrate the elements of a data acquisition system (DAS). Also, explain the roles of function generators, spectral analyzers, harmonic distortion analyzers, and telemetry systems in measurement and monitoring applications. | 10 Marks | L2 | CO4 |
| or |  |  |  |  |  |
| 14. | a. | Explain the application and working principle of LVDT. | 10 Marks | L1 | CO4 |
|  | b. | Explain the working principle and applications of piezoelectric transducers and resistive transducers. How do these transducers differ from each other in terms of their characteristics and usage? | 10 Marks | L1 | CO4 |

| 15. | a. | Explain the working principle of PMMC Instrument by comparing it with a DC motor. | 10 Marks | L2 | CO2 |
| --- | --- | --- | --- | --- | --- |
|  | b. | Explain the working principle of MI Instrument by comparing it with a Induction motor | 10 Marks | L2 | CO2 |
| Or |  |  |  |  |  |
| 16. | a. | A resistance is measured using the voltmeter-ammeter method with direct current (D.C.) excitation. A high-resistance voltmeter is connected directly across the unknown resistance. The voltmeter reading has a maximum possible error of 5%, while the ammeter reading has a maximum possible error of 2%. Solve for maximum possible percentage error in the resistance value derived from these measurements. | 10 Marks | L3 | CO2 |
|  | b. | A variable w is related to three other variables as u=(v*y)/z. The variables are measured with meters of error of 2% of reading, 2% of full scale value and 3% of reading. The actual readings of the three meters are 90, 20 and 50 with 100 being the full scale value for all three. Solve for the maximum uncertainty in the measurement. | 10 Marks | L3 | CO2 |

| 17. | a. | The x deflecting plates of a CRT are 20 mm long and 5 mm apart. The centre of the plate is 25 cm away from the screen. The accelerating voltage is 3000 V. Solve for the deflection sensitivity | 10 Marks | L3 | CO3 |
| --- | --- | --- | --- | --- | --- |
|  | b. | The x deflecting plates of a CRT are 20 mm long and 5 mm apart. The centre of the plate is 25 cm away from the screen. The accelerating voltage is 3000 V. If the length of the trace obtained on the screen is 10 cm, then solve for the rms value of the input voltage. | 10 Marks | L3 | CO3 |
| Or |  |  |  |  |  |
| 18. | a. | A CRO screen has ten divisions on the horizontal scale. If a voltage signal 5 sin(314t+450) is examined with a line base setting of 5 m sec/div, then solve for the number of cycles displayed on the screen. | 10 Marks | L3 | CO3 |
|  | b. | A 312 digit Digital multimeter has the error specification as 0.3% of reading plus 2 counts. . A DC voltage of 10 V is read on its 20 V range. Solve for the % error that can be expressed as a % of the reading | 10 Marks | L3 | CO3 |

***** BEST WISHES *****