| Roll No |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Presidency University

Bengaluru

SCHOOL OF ENGINEERING

Date: 19/07/2024

Time: 9.30 AM to 12.30 PM

Max Marks: 100

Weightage: 50 %

Course Code: ECE 3009

Course Name: TRANSMISSION LINES AND WAVEGUIDES

Program : B.TECH

MAKE UP EXAMINATION JULY 2024

Instructions:

- Read the all questions carefully and answer accordingly.

- Question paper consists of 3 parts.

- Scientific and Non-programmable calculators are permitted.

Part A [Memory Recall Questions]

Answer all the Questions. Each Question carries 02 marks.	                 (5Qx 2M= 10M)

- Give the classification of transmission line?                                [2M] (C.O.No.1) [Knowledge]

- How do you normalize load impedance in Smith chart?	         [2M] (C.O.No.2) [Knowledge]

- Define TE wave?						         [2M] (C.O.No.3) [Knowledge]

- What is phase velocity in waveguide?			         [2M] (C.O.No.4) [Knowledge]

- What is the relationship between reflection coefficient and VSWR of the transmission line?

[2M] (C.O.No.2) [Knowledge]

Part B [Thought Provoking Questions]

Answer all the Questions. Each Question carries 15 marks.                       (2Qx15M=30M)

6. How rectangular waveguide produce an oscillation when it is operating as cavity resonator? Comment on dominant modes in TE and TM modes? Justify your answer with examples for dominant modes?					[15M] (C.O.No.1) [Comprehension]

7. A wave is propagated in a parallel plane waveguide separated by a distance 4cm at a frequency of 4GHz, then what is the dominant mode, cut-off frequency, cut-off wavelength, guide wall wavelength, phase constant?   	  	   [15M] (C.O.No.3) [Comprehension]

Part C [Problem Solving Questions]

Answer all the Questions. Each Question carries 20 marks.	                   (3Qx20M=60M)

8. A load impedance of ZL=10+j20 Ω is required to be matched to a 30 Ω characteristics impedance of the transmission line using a short circuited stub of length ‘l’ connected at a distance d from the load. If the wavelength is 0.5m determine d and l using Smith chart?

[20M] (C.O.No.2) [Application]

9. TE3 mode is propagated through the air between two perfectly conducting planes at a frequency of 2.5GHz. The phase constant at this frequency is measured to be 20 rad/m. Find (i) Cut-off frequency (ii) Separation between the planes (iii) angle of incidence on the plane (iv) Cut-off wavelength?						                  [20M] (C.O.No.3) [Application]

10. If a generator of 0.25V, 2500Hz supplies power to a 5KM transmission line that has the parameters R=1.5 Ω/KM, L=0.00335 H/KM, G=0.08X10-6 mho/KM, C=0.0075 µF/KM. Then what should be the values of ᵞ, λ and Zo of a transmission line?

[20M] (C.O.No.1) [Application