| Roll No |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

PRESIDENCY UNIVERSITY

BENGALURU

SET-B

SCHOOL OF ENGINEERING

END TERM EXAMINATION – MAY/JUNE 2024

| Semester : IV-B.Tech CSE-2022 | Date : JUNE 14-2024 |
| --- | --- |
| Course Code : CSE3081 | Time : 9:30 AM - 12:30 PM |
| Course Name : Digital Image Processing | Max Marks :100 |
| Program : B.Tech | Weightage : 50% |

________________________________________________________________________________________________

Note:	1. Answer ALL 5 FULL Questions.

2. Each Full Question carries 20 Marks

3. Scientific and non-programmable calculator are permitted.

4. Do not write any information on the question paper other than Roll Number.

| 1.a. | Describe the different types of digital images and their primary characteristics. How are these classifications useful in various applications? [Knowledge] | (CO1) | (04 Marks) |
| --- | --- | --- | --- |
| 1.b. | Explain the Electromagnetic spectrum with neat diagram in digital image processing and also give on applications of each spectrum [Comprehension] | (CO1) | (06 Marks) |
| 1.c. | Write a MATLAB script to accomplish the following tasks: [Application] a) Read an RGB image, convert it to a grayscale image, and then convert it to a binary image. b) Read another image of the same size and perform arithmetic operations (addition, subtraction, multiplication, and division) between the two images. Display the resulting images from each operation | (CO1) | (10 Marks) |
| or |  |  |  |
| 2.a. | Explain advantages and disadvantages of digital images. [Knowledge] | (CO1) | (04 Marks) |
| 2.b. | Explain Image deigitization with an neat diagram [Comprehension] | (CO1) | (06 Marks) |
| 2.c. | Solve the following problems: [Application] a) Calculate the storage requirement for a 1024 x 1024 binary image. b) Calculate the storage requirement for a 1024 x 1024 RGB (24-bit) color image. c) A picture with physical dimensions of 2.5 inches by 2 inches is scanned at 150 dpi. Determine the total number of pixels in the resulting image. | (CO1) | (10 Marks) |
| 3.a. | Briefly explain gray level slicing piecewise linear transformation in digital image processing. [Knowledge] | (CO2) | (04 Marks) |
| 3.b. | Using 2nd order derivative, develop a Laplacian Mask, explain in detail. [Comprehension] | (CO2) | (06 Marks) |
| 3.c. | Explain image thresholding and Apply thresholding transformation the given 2D image matrix of size4X4 [Application] 20; 30; 40; 10. 0; 5; 5; 0. 1; 2; 10; 3. 30; 25; 20; 40. | (CO2) | (10 Marks) |
| or |  |  |  |
| 4.a. | Define a histogram and explain the concept of histogram equalization. Why is histogram equalization important in digital image processing? [Knowledge] | (CO2) | (04 Marks) |
| 4.b. | Explain the smoothing of image in frequency domain using [Comprehension] - Ideal low pass filter - Butterworth low pass filter - Gaussian low pass filter | (CO2) | (06 Marks) |
| 4.c. | Explain the process of histogram processing for image enhancement. Discuss the steps involved in histogram equalization and histogram matching. How can these techniques improve the contrast and information content of an image? Provide examples. [Application] | (CO2) | (10 Marks) |
| 5.a. | What are the limitations of using spatial filtering for image restoration, and how can these be overcome [Knowledge] | (CO3) | (04 Marks) |
| 5.b. | List and describe four categories of noise in digital images. [Comprehension] | (CO3) | (06 Marks) |
| 5.c. | Explain a Model of the Image Degradation/Restoration Process [Application] | (CO3) | (10 Marks) |
| or |  |  |  |
| 6.a. | Define noise in digital image and what are the sources of noise. [Knowledge] | (CO3) | (04 Marks) |
| 6.b. | Explain the Gamma noise model with a PDF graph and relevant mathematical equations. [Comprehension] | (CO3) | (06 Marks) |
| 6.c. | Explain the Min-Max Filters in detail with an example [Application] | (CO3) | (10 Marks) |
| 7.a | Write four types of masks to detect lines in digital images [Knowledge] | (CO4) | (04 Marks) |
| 7.b. | Define edge in digital images and write the edge detection using sobel operator both horizontal vertical edges. [Comprehension] | (CO4) | (06 Marks) |
| 7.c | Consider an RGB color image with a pixel represented by the RGB values (128, 128, 128). Convert the RGB pixel to its corresponding HSI values The Hue (H) is typically expressed in radians or degrees. If using degrees, ensure to convert the result the inverse cosine function from radians to degrees. [Application] Answer the following questions: a. What are the normalized RGB values for the given pixel? b. Calculate the Intensity (I) for the pixel. c. Determine the Saturation (S) for the pixel. d. Compute the Hue (H) for the pixel. e. Provide the final HSI values for the pixel. | (CO4) | (10 Marks) |
| or |  |  |  |
| 8.a | Briefly explain Bit-plane piecewise linear transformations in digital image processing. [Knowledge] | (CO2) | (04 Marks) |
| 8.b. | List the categories of noise in digital images based on their probability distribution functions. [Comprehension] | (CO3) | (06 Marks) |
| 8c. | Explain image segmentation by region growing algorithm and perform region growing segmentation on given below 2D image matrix consider pixel 2 and 7 as seed points threshold T = 3.: [Application] | (CO4) | (10 Marks) |
| 9.a | Explain pseudocoloring in image processing and its applications [Knowledge] | (CO4) | (04 Marks) |
| 9.b | Explain four categories of edge detection algorithms in digital image processing. [Comprehension] | (CO4) | (06 Marks) |
| 9.c | Consider a binary image A and a structuring element B defined as follows: [Application] Image A 1; 1; 0; 1. 1; 1; 1; 1. 1; 1; 1; 1. 0; 0; 0; 0. Structuring element B 1. 1. 1. | (CO4) | (10 Marks) |
| or |  |  |  |
| 10.a | Write the classification of image segmentation algorithms. [Knowledge] | (CO4) | (04 Marks) |
| 10.b | Explain the steps of global thresholding algorithm in digital image processing in detail. [Comprehension] | (CO4) | (06 Marks) |
| 10.c | Explain image segmentation by region split and Merge algorithm. perform image segmentation on below given 2D image matrix Threshold value T=3.[Application] | (CO4) | (10 Marks) |