| Roll No |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

PRESIDENCY UNIVERSITY

BENGALURU

SET - B

SCHOOL OF ENGINEERING

END TERM EXAMINATION – MAY/JUNE 2024

| Semester : Semester IV - 2022 | Date : June 13, 2024 |
| --- | --- |
| Course Code : EEE3051 | Time : 09.30am to 01.30pm |
| Course Name : Microcontroller Applications | Max Marks :100 |
| Program : B.Tech. Computer Science and Engineering | Weightage : 50% |

________________________________________________________________________________________________

Note:	1. Answer ALL 5 FULL Questions.

2. Each Full Question carries 20 Marks

3. Scientific and non-programmable calculator are permitted.

4. Do not write any information on the question paper other than Roll Number.

________________________________________________________________________________________________

| 1.a. | The Program Status Word (PSW) in the 8051 microcontroller architecture is a special-purpose register that indicates the current state of the CPU and includes various flags that reflect the results of arithmetic and logical operations. Describe the different bits in the PSW and their functions. [Knowledge] | (CO1) | (08 Marks) |
| --- | --- | --- | --- |
| 1.b. | Show the status of the CY, AC and P flag after the execution of the following program and also calculate the PSW value after each. - ORG 00H MOV A, #38H MUL A, #02FH - CLR C MOV A, #0E7H ADD A, #8DH MOV R6, A [Comprehension] | (CO1) | (12 Marks) |
| or |  |  |  |
| 2.a. | RISC (Reduced Instruction Set Computing) and CISC (Complex Instruction Set Computing) are two distinct computer processor architectures, each characterized by unique design philosophies and the tradeoffs. List the differences between CISC and RISC architectures? [Knowledge] | (CO1) | (08 Marks) |
| 2.b. | Explain the organization and architecture of the internal RAM in the 8051 microcontroller, highlighting addressing scheme, size, and how it interfaces with the CPU and other peripherals [Comprehension] | (CO1) | (12 Marks) |
| 3.a. | Mr. Tony, the microcontroller designer, is programming the timers for mode 2 operation. Help him load the hexadecimal value by computing the value (in hex) that needs to be loaded into TH for each of the following cases; - MOV TH1, #-200 - MOV TH0, #-60 - MOV TH1, #-3 - MOV TH1, #-12 - MOV TH0, #-48 - MOV TH0, #-420 - MOV TH0, #-840 - h. MOV TH0, #-120 [Comprehension] | (CO2) | (08 Marks) |
| 3.b. | You are given an 8051 microcontroller with an array of N bytes stored starting from memory location8100H. Write an assembly language program to sort this array in descending order and explain each step of your program. [Application] | (CO2) | (12 Marks) |
| or |  |  |  |
| 4.a. | Write the ALP using the XRL instruction that can be used to clear the contents of a register by XORing with itself. Show how XRL A, A clears A, assuming that A = 45H [Comprehension] | (CO2) | (08 Marks) |
| 4.b. | An 8051 microcontroller is provided to you, and it has a string of four bytes saved in memory position8100H. To determine which element in this string is the largest, write an assembly language program. Keep the outcome in register A. Describe how your program finds the largest element and explain each step in detail [Application] | (CO2) | (12 Marks) |
| 5.a. | Define the addressing mode of 8051? Explain the various types of addressing modes with suitable examples. [Knowledge] | (CO2) | (08 Marks) |
| 5.b. | You are given a microcontroller with a register A containing an ASCII value of the character 35H and register B containing 34H. Write a program to convert this ASCII value to its equivalent BCD representation and store the result in register B. Explain the theoretical steps involved in converting the ASCII value to BCD and describe how your program accomplishes this conversion. [Application] | (CO2) | (12 Marks) |
| or |  |  |  |
| 6.a. | Describe the 8051 TMOD Register with necessary diagram and also mention the purpose of TMOD register. [Knowledge] | (CO2) | (08 Marks) |
| 6.b. | Write an assembly language program (ALP) to continuously toggle all the bits of port 1 by sending the values 55H and AAH to it. Put a delay between each issuance of data to Port 1. [Application] | (CO2) | (12 Marks) |
| 7.a | With XTAL = 11.0592 MHz, Compute the TH1 value needed to have the following baud rates. - 9600 - 2400 - 1200 - 4800 [Comprehension] | (CO3) | (08 Marks) |
| 7.b. | Write an assembly language program for the 8051 to transfer the letter “R” serially at 4800 bauds, continuously. [Application] | (CO3) | (12 Marks) |
| or |  |  |  |
| 8.a | Explain the Interrupt Enable Register with the necessary diagram. Also, explain the process of enabling and disabling an interrupt [Comprehension] | (CO3) | (08 Marks) |
| 8.b. | Write an assembly language program for the 8051 to transfer “CSE” serially at 9600 baud, 8-bit data, stop bit and display continuously [Application] | (CO3) | (12 Marks) |
| 9.a | Identify and list the potential applications of Internet of Things applied to various fields. [Knowledge] | (CO4) | (08 Marks) |
| 9.b | Explore the role of IoT (Internet of Things) in advancing health monitoring systems. Discuss the various IoT applications employed in health monitoring, such as wearable devices, remote sensors, and smart home technologies. Analyze how these IoT solutions contribute to personalized healthcare management, early detection of health issues, and remote patient monitoring. Provide examples illustrating the effectiveness of IoT in improving healthcare outcomes and enhancing patient engagement [Comprehension] | (CO4) | (12 Marks) |
| or |  |  |  |
| 10.a | Define the Internet of Things (IoT) and discuss its Key Features and Components significance in modern technological advancements. [Knowledge] | (CO4) | (08 Marks) |
| 10.b | Define the term sensor and explain the role of sensors in IoT. Also, list the various types of sensors and their functions. [Comprehension] | (CO4) | (12 Marks) |