| Roll No |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

PRESIDENCY UNIVERSITY

BENGALURU

SET-A

SCHOOL OF ENGINEERING

END TERM EXAMINATION –MAY/JUNE  2024

| Semester : Semester IV - B.Tech CSE - 2022 | Date : JUNE 12 -2024 |
| --- | --- |
| Course Code : MEC3062 | Time : 9:30 AM -12:30 PM |
| Course Name : Hydraulics and Pneumatics | Max Marks :100 |
| Program : B.Tech | Weightage : 50% |

________________________________________________________________________________________________

Note:    1. Answer ALL 5 FULL Questions.

2. Each Full Question carries 20 Marks

3. Scientific and non-programmable calculator are permitted.

4. Do not write any information on the question paper other than Roll Number.

| 1.a. | Mention at least 8 applications of Hydraulics and Pneumatics System. [Knowledge] | (CO1) | (08 Marks) |
| --- | --- | --- | --- |
| 1.b. | A pneumatic cylinder has a bore diameter of 80 mm and a stroke length of 150 mm. The cylinders supplied with compressed air at a pressure of 6 bar (gauge pressure) and operates at a rate of30 cycles per minute. The atmospheric pressure is 1 bar, and the temperature of the air is 20°C.Assume the process is isothermal. 1. Calculate the force exerted by the piston during the extension stroke. 2. Determine the volume of air consumed per minute by the cylinder. 3. Calculate the power required to compress the air to the operating pressure, assuming an isothermal compression process. [Comprehension] | (CO2) | (12 Marks) |
| or |  |  |  |
| 2.a. | Explain the function of speed control of hydraulic motor using flow control valve. [Knowledge] | (CO4) | (08 Marks) |
| 2.b. | a. A hydraulic motor receives a flow rate of 72 LPM at a pressure of 12000 kPa. If the motors peed is 800 RPM and if the motor has a power loss of 3 kW, find the motor actual output torque and overall efficiency. b. A hydraulic motor has a volumetric efficiency of 90% and operates at a speed of 1750 RPM and a pressure of 69 bar. If the actual flow rate consumed by the motor is 0.0047 m3/s and the actual torque delivered by the motor is 147 Nm, find the overall efficiency of the motor. [Comprehension] | (CO3) | (12 Marks) |
| 3.a. | Briefly mention the disadvantages of Non-pressure-compensated flow-control valves. [Knowledge] | (CO2) | (08 Marks) |
| 3.b. | Write a brief note on conditioning and distribution of compressed air. [Comprehension] | (CO1) | (12 Marks) |
| or |  |  |  |
| 4.a. | Draw the schematic diagram of control of a double acting hydraulic cylinder. [Knowledge] | (CO4) | (08 Marks) |
| 4.b. | a. A pneumatic cylinder is required to move a 1000 N load 150mm in 0.5s. What is the output power? b. A pneumatic cylinder is required to extend at a minimum speed of 0.75 m/s in a system with a flow rate of 50 LPM. What cylinder size is required? c. Two cubic meter of a gas at 5 bar and 300 K are compressed to a volume of 1 cubic m and then heated to a temperature of 420 K. what is the new gauge pressure. [Comprehension] | (CO3) | (12 Marks) |
| z |  |  |  |
| 5.a. | Explain the function of pressure relief valve with a neat sketch. [Knowledge] | (CO2) | (08 Marks) |
| 5.b. | A hydraulic motor has a 100 3 cm volumetric displacement. If it has a pressure rating of 140 barand receives oil from a 0.001 3 m/s theoretical flow rate pump, find the motor [Comprehension] (a) speed, (b)theoretical torque, (c) theoretical kW power. | (CO1) | (12 Marks) |
| or |  |  |  |
| 6.a. | With a neat sketch explain the function of hydraulic cylinder sequencing circuit. [Knowledge] | (CO4) | (08 Marks) |
| 6.b. | a. Calculate the required size of the receiver that must supply air to pneumatic system consuming for 10 minutes between 828 kPa and 690 kPa before the compressor resumes operation. b. what size is required if the compressor is running and delivering at 0.170 m3/min [Comprehension] | (CO3) | (12 Marks) |
| 7.a | Explain the function of proportional control valve with a neat sketch. [Knowledge] | (CO2) | (08 Marks) |
| 7.b. | a. A hydraulic motor is required to drive a load at 500 rpm with 1000 Nm of torque. What is the output power? b. A hydraulic motor receives a flow rate of 72 LPM at a pressure of 12000 kPa. If the motor speed is 800 RPM, determine the actual torque delivered by the motor assuming the efficiency100%?[Comprehension] | (CO1) | (12 Marks) |
| or |  |  |  |
| 8.a | What is slip stick effect in pnematic systems? [Knowledge] | (CO4) | (08 Marks) |
| 8.b. | What is the function of a pneumatic actuator? How can you classify the pneumatic actuators? How do hydraulic actuators differ from pneumatic actuators? What is the function of a pneumatic cylinder? [Comprehension] | (CO3) | (12 Marks) |
| 9.a | Explain the function of Pressure reducing valve with a neat sketch. [Knowledge] | (CO2) | (08 Marks) |
| 9.b | A pump has a displacement volume of 98.4 cm3. It delivers 0.0152 m3/s of oil at 1000 RPM and70 bar. If the prime mover input torque is 124.3 Nm. What is the overall efficiency of pump? What is the theoretical torque required to operate the pump? [Comprehension] | (CO1) | (12 Marks) |
| Or |  |  |  |
| 10.a | What is compressor? Mention its classifications. [Knowledge] | (CO3) | (08 Marks) |
| 10.b | The hydraulic pump has a displacement volume of 150 cm³ per revolution and operates at 1200RPM with a pressure of 80 bar. The mechanical efficiency of the hydraulic pump is 85%, and theprime mover input torque is 180 Nm. [Comprehension] 1. Calculate the flow rate delivered by the hydraulic pump in liters per minute (L/min). 2. Determine the hydraulic power output of the pump in kilowatts (kW). 3. Calculate the overall efficiency of the hydraulic pump. | (CO4) | (12 Marks) |