| Roll No |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# PRESIDENCY UNIVERSITY BENGALURU

SET-B

# SCHOOL OF ENGINEERING

END TERM EXAMINATION –MAY/ JUNE 2024

Semester : Semester IV  - 2022

Course Code : PET2002

Course Name : Fundamentals of Geophysical Logging Techniques

Program : B. Tech. Petroleum Engineering

Date : June 12, 2024

Time :9:30AM-12:30 PM

# Max Marks : 100

Weightage : 50%

# Instructions:

- Read all questions carefully and answer accordingly.

- Question paper consists of 3 parts.

- Scientific and non-programmable calculator are permitted.

- Do not write any information on the question paper other than Roll Number.

PART A

ANSWER ANY FIVE QUESTIONS	5QX2M=10

- Tell at least two objectives of well logging.

- Describe Formation Factor.

- Recall the limitations of induction logging.

- Explain the principle of the induction log.

- Mention one key application of the NMR log in reservoir evaluation.

- Mention two key applications of production logging.

- Identify the principle behind using cross-plots in petrophysical analysis.

(CO1) [Knowledge] (CO2) [Knowledge] (CO3) [Knowledge] (CO3) [Knowledge] (CO4) [Knowledge] (CO4) [Knowledge] (CO5) [Knowledge]

PART B

ANSWER ANY FIVE QUESTIONS	5QX10M=50

- Compare and contrast Logging While Drilling (LWD) and Wireline Open Hole Logging in terms of their methodologies, advantages, and limitations.

(CO1) [Comprehension]

- Investigate the effects of borehole invasion on resistivity measurements and propose correction techniques to enhance data accuracy.

(CO2) [Comprehension]

- Analyze the relationship between porosity and resistivity, and discuss how the formation factor is used to quantify this relationship.

(CO2) [Comprehension]

- Formulate a well logging program for a deepwater drilling project, considering the unique challenges and required tools. Explain your choices.

(CO3) [Comprehension]

- Discuss the limitations of sonic logs in highly fractured or unconsolidated formations and suggest how to overcome these limitations.

(CO3) [Comprehension]

- Evaluate the limitations of CBL/VDL logs in highly deviated wells and propose alternative methods for cement evaluation.

(CO4) [Comprehension]

- Evaluate the limitations of Sonic-Neutron cross-plots in highly shaly formations and propose methods

to overcome these challenges.

(CO5) [Comprehension]

PART C

ANSWER ANY TWO QUESTIONS	2QX20M=40

- (a) Calculate the formation temperature at a depth of 3000 meters, given a surface temperature of 20°C and a geothermal gradient of 25°C/km. To calculate the formation temperature, use the formula:

𝑇𝑓 = 𝑇𝑠 + (𝐺g×𝐷), where 𝑇𝑓 = Formation Temperature, 𝑇𝑠 = Surface Temperature, 𝐺g = Geothermal Gradient, 𝐷 = Depth.

Analyze how variations in the geothermal gradient can impact the estimation accuracy. [5+5 marks]

(b) Analyze the relationship between formation resistivity factor (F) and porosity (φ) using Archie's equation, given 𝐹 = 1/𝜑𝑚. Calculate the formation resistivity factor for a formation with a porosity of

0.15 and a cementation exponent (m) of 2.

Evaluate the importance of the cementation exponent in this relationship. [5+5 marks]

NOTE: You may need to correct the given equation.

(CO2) [Application]

- (a) Calculate Porosity (ɸD) from Formation Density Log (ρb) when,

  - ρb = 2.63 g/cc (Sandstone), ρma = 2.75 g/cc (Calcite), and ρf = 1.12 g/cc (Salt Mud)

  - ρb = 2.61 g/cm3 (Sandstone), ρma = 2.75 g/cm3 (Calcite), and ρf = 1.2 g/cm3 (Salt Mud)

  - ρb = 2.31 g/cc (Limestone), ρma = 2.75 g/cc (Calcite)

- Estimate porosity by plotting the given data on Chart 3. Chart 3:

- Compare the output of (a) and (b). [12 + 6 + 2 marks]

(CO3) [Application]

- The geophysical log data for PU Well 4 is shared below. Identify the correct charts and determine cross-plot porosity as well as lithology for the freshwater-invaded zone using the following cross-plot techniques.

- Neutron Porosity – Bulk Density

- Sonic Transit Time – Bulk Density

Compare the results of all data points (i.e. Sl. No. 1 through 8) for both cross-plots as two different cross-plot techniques are used for determining the porosity and lithology of the same formation.

PU Well 4: Geophysical Log Data

Sl. Depth Cal

GR	RHOB NPHI RES_DEEP RES_SHAL RES_MICR DT

DTS

No. (ft)

(inch) (API) (g/cc) (frac) (Ohm_m)	(Ohm_m)	(Ohm_m)    (µs/ft)

| 1 | 616.46 9.21 | 97.17 2.65 | 0.12 | 19.74 | 21.77 | 22.50 | 70.16 158.65 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 623.16 8.79 | 48.66 2.54 | 0.05 | 13.04 | 15.58 | 23.72 | 63.15 115.85 |
| 3 | 626.97 8.56 | 33.51 2.48 | 0.07 | 16.58 | 16.57 | 7.79 | 64.39 110.51 |
| 4 | 637.95 8.68 | 82.60 2.58 | 0.06 | 19.58 | 23.02 | 25.06 | 68.28 148.22 |
| 5 | 643.59 8.63 | 26.04 2.39 | 0.12 | 2.41 | 2.36 | 1.85 | 68.16 113.36 |
| 6 | 647.85 8.60 | 22.92 2.40 | 0.12 | 0.99 | 0.81 | 1.18 | 68.92 121.84 |
| 7 | 653.80 8.60 | 45.37 2.50 | 0.09 | 4.35 | 4.39 | 2.60 | 70.68 134.80 |
| 8 | 668.27 8.90 | 60.00 2.68 | 0.17 | 8.83 | 8.28 | 19.87 | 68.28 137.26 |

Fill out the following Tables with your findings. Table 1:

- Neutron Porosity – Bulk Density Cross-plot

Sl.

No.

1

2

3

4

5

6

7

8

Chart No.	Cross-plot

Porosity

Lithology

Table 2:

- Sonic Transit Time – Bulk Density Cross-plot

Sl.

No.

1

2

3

4

5

6

7

8

Chart No.	Cross-plot

Porosity

Lithology

Charts for cross –plots

Chart 1 :

Chart 2:

Chart 3:

Chart 4:

(CO5) [Application]